EMS智能网关

易事特EMS智能网关EAμEMS-DM206，具有采集与监视、统计分析、优化调度功能，可适用于发电侧、电网侧、用户侧多种场景，为储能行业提供全方位、立体化、系统性的系统解决方案。

EMS Smart Gateway

EAμEMS-DM206 EMS Smart Gateway features acquisition, monitoring, analysis and optimal scheduling. Suitable for generation, grid and user sides, it provides a complete, systematic solution for energy storage.

产品特点

功能丰富

- 64位处理器、ARM架构、主频最高2.0GHz，算力强大，实现区域独立自治

- 策略丰富：支持削峰填谷、需量控制、逆功率保护、计划跟踪、功率平滑等策略4路独立IP网口，适配电站双网及GOOSE专网

- 光储充一体化管理率

易于扩展

- 多种协议支持，支持103、104、IEC61850、GOOSE、MODBUS等各类标准协议接入

- 6路RS485，方便外扩IO、变送器等模块，

- 可选配4G等功能，实现全量设备接入

专业可靠

- 工业级核心板，无风扇免维护设计

- RT-linux系统，毫秒级数据采集处理

- 工业级严苛测试，工作稳定可靠

Product Features

Rich Functions

- 64-bit ARM processor up to 2.0 GHz for high performance & regional autonomous control

- Multiple operating strategies: peak shaving & valley filling, demand control, reverse power protection, schedule tracking, power smoothing, etc.

- 4 independent Ethernet ports for dual-network & GOOSE private network

- Integrated PV, energy storage & EV charging management

Easy Expansion

- Supports standard protocols: IEC 103/104, IEC 61850, GOOSE, MODBUS

- 6×RS485 for flexible IO/transmitter expansion

- Optional 4G for full-device connectivity

Professional & Reliable

- Industrial core board, fanless & maintenance-free

- RT-Linux OS, millisecond-level data acquisition

- Industrial-grade testing for stable operation

技术参数

| 产品参数 |  |
| --- | --- |
| 处理器 | 四核ARM Cortex-A55 |
| 内存 | DDR42GByte |
| 外存 | eMMC 16GByte |
| 操作系统 | RT-linux系统 |
| 接口参数 |  |
| 网 口 | 4路，百兆及千兆以太网口，RJ45接口 |
| HDMI | 1路，HDMI 20A |
| USB | 2路，USB2.0 |
| RS485 | 6路，波特率1200~115200bps |
| CAN | 3路，CAN2.0,波特率最高1000kbps |
| DVDO | 8路DI+8路DO干接点 |
| 环境参数 |  |
| 工作温湿度 | -30℃~+70℃ 0～95%RH(无凝露) |
| 存储温湿度 | -30℃~+70℃ 0～95%RH(无凝露) |

Technical Specifications

| Product Specifications |  |
| --- | --- |
| Processor | Quad-core ARM Cortex-A55 |
| Memory | DDR4: 2 GByte |
| Storage | eMMC: 16 GByte |
| Operating System | RT-Linux OS |
| Interface Parameters |  |
| Network Port | 4× 10/100/1000M Ethernet ports, RJ45 |
| HDMI | 1× HDMI 2.0 |
| USB | 2× USB 2.0 |
| RS485 | 6× serial ports, baud rate: 1200~115200 bps |
| CAN | 3× CAN 2.0, max baud rate: 1000 kbps |
| DVDO | 8× DI + 8× DO dry contacts |
| Environmental Parameters |  |
| Operating Temperature & Humidity | -30℃~+70℃ 0～95%RH(non-condensing) |
| Storage Temperature & Humidity | -30℃~+70℃ 0～95%RH(non-condensing) |